| 1. IDENTIFICATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Product identifier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Product Name |  |  |  | Comtec Dry |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other means of identification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SDS # |  |  |  | QC-512 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Recommended use of the chemical and restrictions on use |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Recommended Use |  |  |  | For industrial use. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Details of the supplier of the safety data sheet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Supplier Address Qual Chem LLC 86 Merz Blvd Akron, OH 44333. |
| Emergency telephone number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Company Phone Number |  |  |  | 1-800-616-CHEM (2436) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Emergency Telephone |  |  |  | INFOTRAC 1-352-323-3500 (International) 1-800-535-5053 (North America) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. HAZARDS IDENTIFICATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Physical state Liquid. |
| Classification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Acute toxicity - Inhalation (Vapors) |  |  |  |  |  |  |  |  |  |  |  |  | Category 4 |  |  |  |  |  |  |
| Serious eye damage/eye irritation |  |  |  |  |  |  |  |  |  |  |  |  | Category 2 |  |  |  |  |  |  |
| Signal Word Warning |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard statements Harmful if inhaled Causes serious eye irritation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Precautionary Statements - Prevention Avoid breathing dust/fume/gas/mist/vapors/spray Use only outdoors or in a well-ventilated area Wash face, hands and any exposed skin thoroughly after handling Wear protective gloves/protective clothing/eye protection/face protection. |
| Precautionary Statements - Response. |
| IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing If eye irritation persists: Get medical advice/attention. |
| IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing Call a poison center or doctor/physician if you feel unwell. |
| Other hazards Harmful to aquatic life with long lasting effects. |
| 3. COMPOSITION/INFORMATION ON INGREDIENTS. Chemical name; CAS No; Weight-%. Proprietary Hydrocarbon; Proprietary; 30-35. Proprietary Quaternary Compound; Proprietary; 10-15. Proprietary Solvent; Proprietary; 5-10. **If Chemical Name/CAS No is "proprietary" and/or Weight-% is listed as a range, the specific chemical identity and/or percentage of composition has been withheld as a trade secret.**. |
| 4. FIRST AID MEASURES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description of first aid measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Eye Contact |  |  | Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. If eye irritation persists: Get medical advice/attention. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Skin Contact |  |  | Wash off immediately with plenty of water for at least 15 minutes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Inhalation |  |  | Remove victim to fresh air and keep at rest in a position comfortable for breathing. Call a poison center or doctor/physician if you feel unwell. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ingestion |  |  | Clean mouth with water and drink afterwards plenty of water. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Most important symptoms and effects, both acute and delayed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Symptoms |  |  | May be harmful if swallowed. May be harmful in contact with skin. Causes mild skin irritation. Harmful if inhaled. Causes serious eye irritation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Indication of any immediate medical attention and special treatment needed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Notes to Physician |  |  | Treat symptomatically. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. FIRE-FIGHTING MEASURES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Suitable Extinguishing Media Use extinguishing measures that are appropriate to local circumstances and the surrounding environment. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Unsuitable Extinguishing Media |  |  | Not determined. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Specific Hazards Arising from the Chemical Not determined. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Protective equipment and precautions for firefighters As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full protective gear. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6. ACCIDENTAL RELEASE MEASURES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Personal precautions, protective equipment and emergency procedures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Personal Precautions |  |  | Use personal protective equipment as required. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Environmental precautions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Environmental precautions |  |  | See Section 12 for additional Ecological Information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Methods and material for containment and cleaning up |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Methods for Containment |  |  | Prevent further leakage or spillage if safe to do so. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Methods for Clean-Up |  |  | Keep in suitable, closed containers for disposal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7. HANDLING AND STORAGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Precautions for safe handling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Advice on Safe Handling |  |  | Avoid breathing dust/fume/gas/mist/vapors/spray. Use only outdoors or in a well-ventilated area. Wash face, hands and any exposed skin thoroughly after handling. Wear protective gloves/protective clothing and eye/face protection. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Conditions for safe storage, including any incompatibilities |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Storage Conditions |  |  | Keep containers tightly closed in a dry, cool and well-ventilated place. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Incompatible Materials |  |  | None known based on information supplied. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8. EXPOSURE CONTROLS/PERSONAL PROTECTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Exposure Guidelines |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chemical name |  |  |  | ACGIH TLV |  |  |  |  | OSHA PEL |  |  |  |  |  |  | NIOSH IDLH |  |  |  |
| Proprietary Solvent |  |  |  | TWA: 20 ppm |  |  |  |  | TWA: 50 ppm TWA: 240 mg/m3 (vacated) TWA: 25 ppm (vacated) TWA: 120 mg/m3 (vacated) S* S* |  |  |  |  |  |  | IDLH: 700 ppm TWA: 5 ppm TWA: 24 mg/m3 |  |  |  |
| Appropriate engineering controls |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Engineering Controls |  |  | Apply technical measures to comply with the occupational exposure limits. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Individual protection measures, such as personal protective equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Eye/Face Protection |  |  | Refer to 29 CFR 1910.133 for eye and face protection regulations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Skin and Body Protection |  |  | Refer to 29 CFR 1910.138 for appropriate skin and body protection. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Respiratory Protection |  |  | Refer to 29 CFR 1910.134 for respiratory protection requirements. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | General Hygiene Considerations |  |  | Handle in accordance with good industrial hygiene and safety practice. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9. PHYSICAL AND CHEMICAL PROPERTIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Information on basic physical and chemical properties |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Physical state |  |  |  | Liquid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Appearance; Not determined. | Odor; Not determined. |
| Color; Not determined. | Odor Threshold; Not determined. |
| Property; Values. | Remarks • Method. |
| pH; Not determined. |  |
| Melting point / freezing point; Not determined. |  |
| Boiling point / boiling range; Not determined. |  |
| Flash point; Not determined. |  |
| Evaporation Rate; Not determined. |  |
| Flammability (Solid, Gas); Liquid-Not applicable. |  |
| Flammability Limit in Air. |  |
| Upper flammability or explosive limits; Not determined. |  |
| Lower flammability or explosive limits; Not determined. |  |
| Vapor Pressure; Not determined. |  |
| Vapor Density; Not determined. |  |
| Relative Density; Not determined. |  |
| Water Solubility; Not determined. |  |
| Solubility in other solvents; Not determined. |  |
| Partition Coefficient; Not determined. |  |
| Autoignition temperature; Not determined. |  |
| Decomposition temperature; Not determined. |  |
| Kinematic viscosity; Not determined. |  |
| Dynamic Viscosity; Not determined. |  |
| Explosive Properties |  |  |  | Not determined |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Oxidizing Properties |  |  |  | Not determined |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10. STABILITY AND REACTIVITY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reactivity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Not reactive under normal conditions.. |
| Chemical stability Stable under recommended storage conditions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Possibility of hazardous reactions None under normal processing. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Conditions to Avoid Keep out of reach of children. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Incompatible materials None known based on information supplied. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazardous decomposition products None known based on information supplied. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11. TOXICOLOGICAL INFORMATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Information on likely routes of exposure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Product Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Eye Contact |  |  | Avoid contact with eyes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Skin Contact |  |  | May be harmful in contact with skin. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Inhalation |  |  | Harmful if inhaled. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ingestion |  |  | May be harmful if swallowed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Component Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chemical name |  |  | Oral LD50 |  |  |  |  | Dermal LD50 |  |  |  |  |  |  | Inhalation LC50 |  |  |  |  |
| Proprietary Hydrocarbon |  |  | = 7400 mg/kg ( Rat ) |  |  |  |  | > 2000 mg/kg ( Rabbit ) |  |  |  |  |  |  | = 4.6 mg/L ( Rat ) 4 h |  |  |  |  |
| Proprietary Quaternary Compound |  |  | = 960 mg/kg ( Rat ) |  |  |  |  | - |  |  |  |  |  |  | - |  |  |  |  |
| Proprietary Solvent |  |  | = 470 mg/kg ( Rat ) |  |  |  |  | = 435 mg/kg ( Rabbit ) |  |  |  |  |  |  | = 486 ppm ( Rat ) 4 h = 450 ppm ( Rat ) 4 h |  |  |  |  |
| Proprietary Solvent |  |  | = 20 g/kg ( Rat ) |  |  |  |  | = 20800 mg/kg ( Rabbit ) |  |  |  |  |  |  | - |  |  |  |  |
| Proprietary Solvent |  |  | = 5660 µL/kg ( Rat ) = 1900 mg/kg ( Rat ) |  |  |  |  | = 3100 mg/kg ( Rabbit ) |  |  |  |  |  |  | - |  |  |  |  |
| Proprietary Alcohol |  |  | = 1400 mg/kg ( Rat ) = 1378 mg/kg ( Rat ) |  |  |  |  | > 2 g/kg ( Rabbit ) |  |  |  |  |  |  | - |  |  |  |  |
| Symptoms related to the physical, chemical and toxicological characteristics |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Symptoms |  |  | Please see section 4 of this SDS for symptoms. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Delayed and immediate effects as well as chronic effects from short and long-term exposure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Skin corrosion/irritation |  |  | Causes mild skin irritation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Serious eye damage/eye irritation |  |  | Causes serious eye irritation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Carcinogenicity |  |  | Group 3 IARC components are "not classifiable as human carcinogens". |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chemical name |  | ACGIH |  |  |  | IARC |  |  |  |  | NTP |  |  |  |  |  | OSHA |  |  |
| Proprietary Solvent |  | A3 |  |  |  | Group 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Legend |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ACGIH (American Conference of Governmental Industrial Hygienists) A3 - Animal Carcinogen |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IARC (International Agency for Research on Cancer) Group 3 IARC components are "not classifiable as human carcinogens" |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Numerical measures of toxicity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The following values are calculated based on chapter 3.1 of the GHS document . |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Oral LD50 |  |  | 3,957.82 mg/kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dermal LD50 |  |  | 3,397.80 mg/kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ATEmix (inhalation-dust/mist) |  |  | 8.60 mg/L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ATEmix (inhalation-vapor) |  |  | 13.47 mg/L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12. ECOLOGICAL INFORMATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ecotoxicity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Harmful to aquatic life with long lasting effects. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Component Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chemical name |  |  | Algae/aquatic plants |  |  |  |  | Fish |  |  |  |  |  |  | Crustacea |  |  |  |  |
| Proprietary Hydrocarbon |  |  |  |  |  |  |  | 10000: 96 h Pimephales promelas mg/L LC50 static 35: 96 h Pimephales promelas mg/L LC50 flow-through |  |  |  |  |  |  |  |  |  |  |  |
| Proprietary Solvent |  |  |  |  |  |  |  | 2950: 96 h Lepomis macrochirus mg/L LC50 1490: 96 h Lepomis macrochirus mg/L LC50 static |  |  |  |  |  |  | 1000: 48 h Daphnia magna mg/L EC50 1698 - 1940: 24 h Daphnia magna mg/L EC50 |  |  |  |  |
| Proprietary Solvent |  |  | 19000: 96 h Pseudokirchneriella subcapitata mg/L EC50 |  |  |  |  | 51400: 96 h Pimephales promelas mg/L LC50 static 710: 96 h Pimephales promelas mg/L LC50 41 - 47: 96 h Oncorhynchus mykiss mL/L LC50 static 51600: 96 h Oncorhynchus mykiss mg/L LC50 static |  |  |  |  |  |  | 1000: 48 h Daphnia magna mg/L EC50 Static 10000: 24 h Daphnia magna mg/L EC50 |  |  |  |  |
| Persistence/Degradability Not determined. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bioaccumulation There is no data for this product. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mobility |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chemical name |  |  |  |  |  |  |  | Partition coefficient |  |  |  |  |  |  |  |  |  |  |  |
| Proprietary Solvent |  |  |  |  |  |  |  | 0.81 |  |  |  |  |  |  |  |  |  |  |  |
| Other Adverse Effects Not determined |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13. DISPOSAL CONSIDERATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Waste Treatment Methods |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Disposal of Wastes |  |  | Disposal should be in accordance with applicable regional, national and local laws and regulations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Contaminated Packaging |  |  | Disposal should be in accordance with applicable regional, national and local laws and regulations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14. TRANSPORT INFORMATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note |  |  |  | Please see current shipping paper for most up to date shipping information, including exemptions and special circumstances. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DOT |  |  |  | Not regulated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IATA |  |  |  | Not regulated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IMDG |  |  |  | Not regulated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15. REGULATORY INFORMATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| International Inventories |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chemical name |  | TSCA | TSCA Inventory Status |  | DSL/NDSL |  | EINECS/ELINCS |  |  | ENCS |  | IECSC |  | KECL |  |  |  | PICCS | AICS |
| Proprietary Hydrocarbon |  | X | ACTIVE |  | X |  | X |  |  |  |  | X |  | X |  |  |  | X | X |
| Proprietary Quaternary Compound |  | X | ACTIVE |  | X |  | X |  |  |  |  | X |  | X |  |  |  | X | X |
| Proprietary Solvent |  | X | ACTIVE |  | X |  | X |  |  | X |  | X |  | X |  |  |  | X | X |
| Proprietary Solvent |  | X | ACTIVE |  | X |  | X |  |  | X |  | X |  | X |  |  |  | X | X |
| Proprietary Amines |  | X | ACTIVE |  | X |  |  |  |  |  |  | X |  |  |  |  |  |  | X |
| Proprietary Alcohol |  | X | ACTIVE |  | X |  |  |  |  |  |  | X |  | X |  |  |  | X | X |
| Proprietary Solvent |  | X | ACTIVE |  | X |  | X |  |  | X |  | X |  | X |  |  |  | X | X |
| Legend: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | TSCA - United States Toxic Substances Control Act Section 8(b) Inventory |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | DSL/NDSL - Canadian Domestic Substances List/Non-Domestic Substances List |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EINECS/ELINCS - European Inventory of Existing Chemical Substances/European List of Notified Chemical Substances |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ENCS - Japan Existing and New Chemical Substances |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IECSC - China Inventory of Existing Chemical Substances |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | KECL - Korean Existing and Evaluated Chemical Substances |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PICCS - Philippines Inventory of Chemicals and Chemical Substances |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | AICS - Australian Inventory of Chemical Substances |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| US Federal Regulations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CERCLA This material, as supplied, does not contain any substances regulated as hazardous substances under the Comprehensive Environmental Response Compensation and Liability Act (CERCLA) (40 CFR 302) or the Superfund Amendments and Reauthorization Act (SARA) (40 CFR 355). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SARA 313 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chemical name |  |  |  |  |  | CAS No |  |  |  |  | Weight-% |  |  |  |  |  | SARA 313 - Threshold Values % |  |  |
| Proprietary Solvent - |  |  |  |  |  |  |  |  |  |  | 5-10 |  |  |  |  |  | 1.0 |  |  |
| CWA (Clean Water Act) This product does not contain any substances regulated as pollutants pursuant to the Clean Water Act (40 CFR 122.21 and 40 CFR 122.42) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| US State Regulations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| California Proposition 65 This product does not contain any Proposition 65 chemicals. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| U.S. State Right-to-Know Regulations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chemical name |  |  | New Jersey |  |  |  |  | Massachusetts |  |  |  |  |  |  | Pennsylvania |  |  |  |  |
| Proprietary Solvent |  |  | X |  |  |  |  | X |  |  |  |  |  |  | X |  |  |  |  |
| Proprietary Solvent |  |  | X |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| 16. OTHER INFORMATION. NFPA.; Health Hazards Not determined.; Flammability Not determined.; Instability Not determined.; Special Hazards Not determined.. HMIS.; Health Hazards Not determined.; Flammability Not determined.; Physical hazards Not determined.; Personal Protection Not determined.. Issue Date:; 01-Oct-2019. Revision Date:; 03-Oct-2019. Revision Note:; New format. Disclaimer The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process, unless specified in the text.. End of Safety Data Sheet. |